PC-Download (PCDN) by Dave Baldwin, 2/98.

PC-Download is a highly modified version of a program called

Tinyterm by Michael Brandon.  The original documentation for Tiny-

term is included here and it applies to PCDN except as noted. '|'

is placed at the beginning of changed lines.

PCDN is intended for high speed downloads from a PC running a

host program such ProComm.  PCDN requires hardware (rts/cts) hand-

shaking.  On a 4 MHz Z80, PCDN will operate at 19.2kb.

-----------------------------------------------------------------

Documentation for Tinyterm v1.0. Written by Michael Brandon.

Tinyterm - a tiny terminal program

----------------------------------

Tinyterm is a tiny terminal program which will allow you

to  communicate  with  other  computers via a modem or an RS-232

connection, and to download files using the XMODEM file transfer

protocol.  The  reason  I  wrote  Tinyterm is that I had a crude

terminal  program  which  would  allow  me to communicate with a

Bulletin Board System, but not to  download  files; Tinyterm can

thus  be  used  to  download  either source or object code for a

sophisticated terminal program such as MODEM7A or YAM.

How to get PCDN (Tinyterm) running on your system

------------------------------------------

|        You will need a Z-80 macro assembler, and either:

(1)  A friend with a  computer  who has (or can get hold

of) a copy of the source code for Tinyterm

OR (2)  A hardcopy listing of Tinyterm

There are two versions of the source code for Tinyterm -

one written in Intel mnemonics (for an 8080 assembler),  and one

in Zilog mnemonics (for a Z-80 assembler). They are on the files

"TINYTERM.ASM" and  "TINYTERM.MAC" respectively. Remember to get

the right version!

| There is only a Z80 version of PCDN.

In case (1),  you  can  download  the  source  code  and

documentation  for  Tinyterm from your friend's computer  via  a

modem or a direct RS-232 connection using STAT and PIP (for CP/M

1.x and 2.x) or DEVICE and  PIP  (for  CP/M  3.0). The files are

small enough that most CP/M systems with a reasonable amount  of

memory will be able to download the files.

In case (2),  you  will have to type in the source code

by hand (minus the comments of course!). This isn't as bad as it

seems  however,  since  there are only about 260 lines of actual

code.

Having procured  the source code for Tinyterm, there are

some  modifications  which will have to be made to it before  it

will  run  on your system. The following constants will probably

have to be changed -

|mod_data   -  this  is  the Z-80 I/O port through  which

your SIO, UART or USART data register  (and  hence  your

modem)  is accessed. You should  be  able  to  find  the

correct  value  for  this  by  looking  at the Technical

Manual for your CP/M system.

|mod_status  -  this  is the Z-80 I/O port through  which

your  SIO, UART OR USART status  register  is  accessed.

Look in your Technical Manual for this too.

rx_mask    -  this  is  the  'mask'  which extracts the "receive

ready" information from the SIO,  UART  or  USART status

byte.  It  is  'AND'ed with the status byte to determine

whether  or not a character has  been  received  by  the

modem.

rx_ready   -  the  value  of  "rx_mask  AND  status byte" when a

character  has been received by the modem and is waiting

to be read.

tx_mask    -  this  is  the  'mask' which extracts the "transmit

ready" information from the SIO,  UART  or  USART status

byte.  It  is  'AND'ed with the status byte to determine

whether  or  not  the  modem  is  ready  to  transmit  a

character.

tx_ready   -  the  value  of  "tx_mask AND status byte" when the

modem is ready to transmit a character.

cpu_speed  - this is the speed of your CPU in kHz.  The value is

not very critical.

Note : the values for rx_mask, rx_ready, tx_mask and tx_ready as

they stand are correct for a Z-80 SIO; thus, they will only have

to  be  changed  if  you are not using a Z-80 SIO to access your

modem.

| The rts/cts handshaking code will also have to be changed.

You may also  have  to  add  code  to the program, where

indicated  in  the  source  code,   to   set   up   the  correct

communications parameters, if they are not already  set  up. The

correct parameters are - 300 baud, 8 data bits, 1  stop  bit, no

parity. Your system may default to this setup, or you may have a

system utility to do this from CP/M, or you may be able to do it

via hardware jumper leads etc. Add code to the program to set up

the correct parameters only if you have to.

| The baud rate is defined by equates at the beginning of the

| source code.  Equates for 19.2k, 9600, and 4800 are included.

Once you  have  made  the required changes to the source

code, you should be able to assemble the source code and run the

program.

Using Tinyterm

--------------

To run Tinyterm,  just  type "tinyterm <filename>" where

<filename>  is  the  name of the file  which  is  to  contain  a

downloaded file. To make the program simple, I decided  to  make

specifying  the file name compulsory; however, if no downloading

is done then the file name specified is not used. Having started

the  program running, your computer should now behave as a full-

duplex  terminal  i.e. any character you type will be  sent  out

through  the  modem  (but  not  displayed),  and  and  character

received  from  the  modem  is  displayed  on  the  screen.  Two

characters  have  a  special  meaning  to  Tinyterm  - they  are

<control> C and <control> R. When  <ctrl>  C  is typed, Tinyterm

exits to CP/M; when <ctrl> R is typed, a file is downloaded.

Downloading files using Tinyterm

--------------------------------

To download  a  file  using  Tinyterm, you should ensure

that  the  other  computer  with  which you are communicating is

ready  to  transmit  a  file  using the XMODEM (alias CP/M alias

CHRISTENSEN)  protocol. Typing "<control>  R"  starts  the  file

download  process.  If for instance you ran Tinyterm  by  typing

"tinyterm fred", then  when you type "<control> R", any existing

file called "fred" is deleted, and the downloaded file  will  be

put  onto  the file "fred". Note  that  only  one  file  can  be

downloaded at a time - if you download two files during  the one

execution  of Tinyterm, then the first file downloaded  will  be

lost.  To  download  more  than  one  file,  you  will  have  to

repeatedly run the program, download a file, and exit to CP/M.

Tinyterm   will   print   various  messages  during  the

downloading of a file, the least offensive of which are -

"block received"

A block was correctly received.

"error in block"

Part of the block got garbled.

"error in block number"

The block number got garbled.

"file transfer completed"

The file has been successfully downloaded; your computer

now acts as a terminal again.

"no <soh> or <eot>"

The start-of-block indicator got garbled.

"timeout"

The  computer with which you are communicating  did  not

respond within 10 seconds.

"(repeated block)"

Your  acknowledgement  for a block got garbled,  so  the

computer  with  which  you  are communicating resent the

current block.

The following  messages  indicate  a more serious error;

the  file  download  is  aborted, and your computer goes back to

acting like a terminal.

"aborted"

Ten  consecutive  errors were encountered, so  the  file

download is aborted.

"can't close file"

The disc file cannot be closed. What to do  about this I

don't know.

"can't create file"

The  disc  file cannot be created. Perhaps the  disc  is

write protected.

"disc limit reached"

The disc to which you are writing is full.